UX/UI Guidelines 

This document compiles the guidelines that the UX/UI team will follow to design and create an interface that will best answer to the various requirements of the users.

We will first determine the KPIs that can be derived from the available machine variables.

There are numbers of variables that already allow us to monitor different aspects of the machines and some others can be used to calculate more complex KPIs.

There are three types of indicators that we have grouped in the following way : 
1. The Operational KPIs
2. The Quality/Downtime KPIs
3. The Maintenance/Health KPIs


The Operational KPIs :

| KPI Name | Description | How to Calculate |
| --- | --- | --- |
| Machine Availability (%) | % of time the machine is not in alarm, standby, or off. | Available time / Total time * 100 where Available time = total duration when MACHINE_IN_OPERATION = true or OP_MODE_OPERATIONAL_CONTROL = true. |
| Operating Time (h) | Total hours the machine was running a program. | Sum of time intervals where PROG_RUN = true. |
| Standby Time (h) | Time when the machine was on but not executing programs. | Time when OP_MODE_STANDBY = true. |
| Program Execution Count | Number of executed machining programs. | Count transitions where PROG_RUN goes from false → true. |
| Average Program Duration (min) | Mean duration of each program run. | (Σ Program end time - start time) / Number of programs. |

The Quality KPIs :

| KPI Name | Description / Meaning | How to Calculate |
| --- | --- | --- |
| Alarm Frequency | Number of alarm events in a period. | Count transitions where ALARMA_ACTIVA = true. |
| Mean Time Between Alarms (MTBA) | Average time between alarms. | Operating time / Number of alarms. |
| Mean Time To Repair (MTTR) | Avg recovery time from alarm state to operational. | Avg(time when ALARMA_ACTIVA = true → next MACHINE_IN_OPERATION = true). |
| Emergency Stop Count | Number of emergency button activations. | Count transitions where MACHINE_EMERGENCY = true or EMERGENCY_PULSED = true. |
| Power Failure Count | Number of unexpected shutdowns. | Use POWER_OFF_FAIL_COUNTER. |
| Program Interruption Frequency | Frequency of interrupted programs. | Count of PROG_INTERRUPTED = true. |

The Maintenance/Health KPIs :

| KPI Name | Description / Meaning | How to Calculate |
| --- | --- | --- |
| Motor Temperature Avg (°C) | Mean temperature of motors (X/Y/Z/Spindle). | avg(TEMPERATURA_MOTOR_X, Y, Z, MANDRINO_TEMP) over operational time. |
| Warming Cycle Count | Total warming cycle executions. | Sum of all CYCLE_WARMING_* counters. |
| Gear Change Count | Number of spindle gear changes. | CAMBIO_GAMA_CONTADOR total. |
| Average Gear Change Time (s) | Avg time spent changing gear. | avg(CAMBIO_GAMA_TIEMPO_CAMBIO_UD). |

We can create different tabs for each specific type of indicator.These indicators mainly use variables directly present in the dataset, some need a little bit more advanced calculations. 

Here is the repartition of the different indicators according to the group they refer to. Each type of monitoring will appear in a specific tab.

Home Page

For a more impacting dashboard, we have determined a few KPIs that will give a general overview of the activity and its performance.
The Home page will be a dashboard containing these following indicators : 

Indicators presented as numbers (% or corresponding units) : 

1. OEE = Availability × Performance × Quality. Measures overall equipment effectiveness

2. Availability = (Operating Time / Planned Production Time) × 100. Accounts for downtime losses including breakdowns and changeovers

3. Performance = (Ideal Cycle Time × Total Count / Operating Time) × 100. Measures speed losses and minor stoppages

4. Machine Utilization Summary (Current Shift) :

- Planned Time

- Run Time

- Idle Time

- Down Time

Possible visual component : compare each indicator to a target value and display them in a representative color (red, orange, green) according to its value.
Hover tooltip showing details like ???

Indicators presented as Graphs : 

1. Machine Status Timeline 
Visual components : bar chart with colors according to the status and height according to the time period
zooming and panning for detailed inspection of specific time windows

Operation Monitoring 

This tab will compile the KPIs referred to as “Operational KPIs”.
The idea is to select the best way to display the indicators in order to maximize their visual clarity to a user.

Indicators presented as numbers (% or corresponding units) : 

1. Program Execution Count : 
Column chart filtered by day or shift or machine (Each column = number of programs completed)

2. Average Program Duration
Box plot filtered per machine or per program type 

Possible visual component : compare each indicator to a target value and display them in a representative color (red, orange, green) according to its value.

Indicators presented as Graphs : 

1. Operating Time : 
Stacked Bar Chart filtered either by day, shift or machine
Possible visual component : use of color bands (green = operating, gray = powered off/powered on but not operating)

2. Standby Time :
Combined to operating time in one chart ? 
Possible visual component : add color (red for standby) to the stacked bar chart 
Pie chart to display standby time/shift time period

Energy Monitoring 

This section’s goal is to compile indicators related to Energy.
Allow operators, engineers, and managers to understand how energy is used by each machine, program, and over time to detect waste, optimize performance, and correlate energy with production.

Indicators presented as numbers : 

(see how to calculate these following indicators)

The top section consists of 3 to 4 cards with numeric indicators like : 
- Total energy consumption (kWh)
- Total energy consumption per Operating hour (kWh/h)
- Energy per Program (kWh/program)

Possible visual component : Color-code KPI cards (green = efficient, orange = normal, red = high consumption)
We can add more detailed charts in the middle section.

1. For example, the total energy consumption over time can be displayed in a line chart
X axis : time
Y axis : energy

Possible visual component : add interactive filters like per machine or per program

2. To compare the energy consumption between the different programs, we can create a Vertical bar chart so the most energy-heavy program can be clearly shown


Alert Monitoring